Instituto Tecnológico

y de Estudios Superiores de Occidente.

Reconocimiento de validez oficial de estudios de nivel superior según acuerdo secretarial 15018, publicado en el Diario Oficial de la Federación del 29 de noviembre de 1976.

Departamento de Electrónica, Sistemas e Informática

Maestría en Sistemas Computacionales

# Deep Learning Sensor Fusion of a MARG IMU and GPS Velocity for 3-D Pose Estimation

Trabajo recepcional que para obtener el grado de

Maestro en Sistemas Computacionales

Presenta: Ing. Abraham Rodríguez Vázqiez

Asesor: Mtro. Martín Alonso Sinsel Duarte

Tlaquepaque, Jalisco. Septiembre de 2026.

ACKNOWLEDGEMENTS

I thank Eng. Alberto Arjona, for having been a great source of inspiration and guidance at the beginning of my career in the software industry.

I thank M.Sc. Martín Sinsel, for his patience and constancy, and for having believed in me, even though I was coming from outside the field of electronic engineering.

I thank ITESO, for having instilled in me a spirit of service, since with time, I have come to see that there is no better way to discover, to grow and to have an influence, than by serving.

AGRADECIMIENTOS

Agradezco al Ing. Alberto Arjona, por haber sido de gran inspiración y guianza en los inicios de mi carrera en la industria del software.

Agradezco al Mtro. Martín Sinsel, por su paciencia y constancia, y por haberme tenido fe, aunque fuese ajeno al área de ingeniería electrónica.

Agradezco al ITESO, por haber embebido en mí, el espíritu de servicio, ya que, tiempo después, me consta que no hay mejor forma de descubrir, crecer e influir, que sirviendo.

DEDICATION

I dedicate this thesis to my daughter Nathalie, for having taught me that life is a gift.

Dedicatoria

Dedico la presente tesis a mi hija Nathalie, por haberme enseñado que la vida es un regalo.

ABSTRACT

This thesis presents a deep learning approach to the sensor fusion of a magnetic, angular-rate and gravity (MARG) inertial unit and a GPS velocity, with the aim of estimating a 3-D position and a unit quaternion without retrieving the process and measurement noise of a Kalman filter, and without feeding the network a ground-truth previous pose. A one-second window of accelerometer, gyroscope, magnetometer and GPS-velocity measurements is mapped by a dual-head temporal convolutional network to a residual velocity and a short-horizon orientation increment; those increments are then integrated and compared, on the same simulated LoggedQuadcopter stream, with MathWorks' insfilterMARG extended Kalman filter, using absolute and relative trajectory error and a mean quaternion distance. On a chronological TEST holdout the learned estimator reaches 1.12 m ATE, 0.31 m RTE and 4.95°, i.e. it beats the filter on orientation (7.46°) and ties open-loop GPS-velocity integration on ATE (1.10 m), while the filter remains better on position, as expected, because it is still given GPS latitude, longitude and altitude, which the network never sees. A recursive closed-loop network and a hybrid residual on the frozen EKF were implemented as design alternatives and are not the defended method. The work is simulated and limited to a single trajectory; Chapter 2 surveys the related techniques and the three-way fork, Chapter 4 describes the increment pipeline, Chapter 5 reports the comparison, and Chapter 6 states the conclusions and the recursive line as future work.

RESUMEN

Esta tesis presenta un enfoque de aprendizaje profundo para la fusión de un sensor inercial MARG (magnético, de velocidad angular y de gravedad) y de una velocidad GPS, con el fin de estimar una posición 3D y un cuaternión unitario, sin recuperar las covarianzas de ruido de un filtro de Kalman y sin alimentar a la red con la pose verdadera anterior. Una ventana de un segundo de acelerómetro, giroscopio, magnetómetro y velocidad GPS es mapeada por una red convolucional temporal de dos cabezas a una velocidad residual y a un incremento de orientación de corto horizonte; esos incrementos se integran y se comparan, sobre la misma trayectoria simulada LoggedQuadcopter, con el filtro de Kalman extendido insfilterMARG de MathWorks, mediante el error absoluto y relativo de trayectoria y una distancia media de cuaterniones. En una partición cronológica de prueba, el estimador aprendido alcanza 1.12 m de ATE, 0.31 m de RTE y 4.95°, es decir, supera al filtro en orientación (7.46°) y empata la integración en lazo abierto de la velocidad GPS en ATE (1.10 m), mientras que el filtro sigue siendo mejor en posición, como es de esperarse, porque aún recibe latitud, longitud y altitud, que la red nunca ve. Una red recursiva en lazo cerrado y un residual híbrido sobre el EKF congelado se implementaron como alternativas de diseño y no constituyen el método que se defiende. El trabajo es simulado y se limita a una sola trayectoria; el Capítulo 2 revisa las técnicas relacionadas y la bifurcación de tres opciones, el Capítulo 4 describe el pipeline de incrementos, el Capítulo 5 reporta la comparación y el Capítulo 6 plantea las conclusiones y la línea recursiva como trabajo futuro.

TABLA DE CONTENIDO

[Dar clic en el menú de Referencias/ Dar clic en Tabla de Contenido/ Seleccionar Tabla de Contenido Personalizada/ Seleccionar Formato Formal, 4 Niveles, Dar Clic en Opciones. En la ventana de Opciones, asignar Nivel de TDC 1 a Título Intro. Dar clic en Aceptar, y de nuevo en Aceptar].

Maestría en Sistemas Computacionales	1

1.	INTRODUCCIÓN	12

1.1.	Antecedentes	13

1.2.	Justificación	13

1.3.	Problema	13

1.4.	Hipótesis	14

1.5.	Objetivos	14

1.5.1.	Objetivo General:	14

1.5.2.	Objetivos Específicos:	14

1.6.	Novedad científica, tecnológica o aportación	14

2.	ESTADO DEL ARTE o de la TÉCNICA	15

2.1.	Classical GNSS/INS integrated navigation	15

2.2.	Learning-based inertial odometry	16

Hybrid learning and filtering	16

Deep learning for GNSS/INS outage aiding	16

Synthesis and the three design options	17

3.	MARCO TEÓRICO/CONCEPTUAL	18

3.1.	Concepto básico 1	19

3.2.	Esquema básico 2	19

3.2.1.	Si se requiere subtema	19

3.2.1.1.	Subsubtema	19

4.	DESARROLLO METODOLÓGICO	20

4.1.	Design alternatives for the learned estimator	20

Simulated dataset and the classical baseline	21

Defended estimator: increment TCN	21

Recursive and hybrid pilots	21

5.	RESULTADOS Y DISCUSIÓN	23

5.1.	Resultados	23

5.2.	Discusión	24

6.	CONCLUSIONES	25

6.1.	Conclusiones	25

6.2.	Trabajo Futuro	25

LISTA DE FIGURAS

Figura 1. Ventana que se abre para insertar una imagen.	2

[En el menú de Referencias, seleccionar Insertar Tabla de Ilustraciones. Si se desea insertar para Figuras, solo se debe seleccionar el rótulo de Figura.]

LISTA DE TABLAS

Tabla 1. Presentación de información en tablas.	2

[En el menú de Referencias, seleccionar Insertar Tabla de Ilustraciones. Si se desea insertar para Tablas, solo se debe seleccionar el rótulo de Tabla.]

LISTA DE ACRÓNIMOS Y ABREVIATURAS

| ATE |  | Absolute Trajectory Error |
| --- | --- | --- |
| CNN |  | Convolutional Neural Network |
| DL |  | Deep Learning |
| EKF |  | Extended Kalman Filter |
| GNSS |  | Global Navigation Satellite System |
| GPS |  | Global Positioning System |
| GT |  | Ground Truth |
| IMU |  | Inertial Measurement Unit |
| INS |  | Inertial Navigation System |
| KF |  | Kalman Filter |
| LLA |  | Latitude, Longitude, Altitude |
| LSTM |  | Long Short-Term Memory |
| MARG |  | Magnetic, Angular Rate, and Gravity |
| MLP |  | Multilayer Perceptron |
| MSE |  | Mean Squared Error |
| NED |  | North-East-Down |
| RMSE |  | Root Mean Square Error |
| RTE |  | Relative Trajectory Error |
| TCN |  | Temporal Convolutional Network |
| ZOH |  | Zero-Order Hold |

[5 SALTOS DE LÍNEA CON ESTILO NORMAL ANTES DE PONER EL TITULO]

# INTRODUCCIÓN

Resumen: This chapter frames the problem of fusing a magnetic, angular-rate and gravity (MARG) inertial unit with a consumer GNSS receiver, states why a learned increment model is worth writing down next to a classical extended Kalman filter, and records the objectives and the novelty claim that the rest of the document has to defend.

## Antecedentes

As with many sensing problems that used to live entirely inside a filter designer’s notebook, inertial/GNSS fusion has gone through a transitional period in which the same job, i.e. turning noisy accelerometers, gyroscopes and satellite fixes into a pose, is now also attempted with sequence models that never write down a process-noise matrix; the classical line is still the extended Kalman filter and its error-state cousins, of which MathWorks’ insfilterMARG is the concrete baseline used in this work, while the learning line, from RIDI and IONet through RoNIN and OriNet, replaces double integration with a windowed regression of displacement or velocity [1]–[4]. A third family keeps the filter and trains a network to supply a residual, a covariance, or a pseudo-GNSS measurement, which is the pattern of TLIO, AI-IMU, Al-Sharman and Wang [5]–[8]. This thesis sits in that landscape with a full MARG suite plus GPS velocity, a 3-D position and quaternion target, and a simulated stream generated by the same insfilterMARG example that later serves as the numerical opponent; the earlier course project that fed a ground-truth previous pose into the net is treated here as a rejected setup, not as the method to defend.

## Justificación

The practical pressure is cost and time: a well-tuned EKF can be excellent on a given IMU, and it can also become a week of Q and R archaeology the moment the sensor, the mounting, or the motion statistics change, whereas a learned increment model, if it works, is a promise that those variances do not have to be retrieved by hand. I consider that the more interesting question for a master’s thesis is not whether deep learning can be stapled onto a Kalman filter, that path is already the 2020–2025 paper farm [5]–[8], but whether a Q/R-free network that never sees the true pose can be compared, honestly, with the filter that generated its training noise, on the same trajectory and with the same metrics the inertial-odometry literature already uses, i.e. absolute and relative trajectory error plus a quaternion distance [3], [4]. There is also a narrower, local justification: the hardware side of this work, an ESP32 logging a BNO055 and a NEO-M8N, still needs a fusion story that does not collapse the first time the process-noise table is copied from a datasheet.

## Problema

The practical problem is to estimate a 3-D position and a unit quaternion from a 160 Hz MARG stream and a 1 Hz GPS velocity, without assuming that the previous true pose will be available at runtime. The scientific problem is to do that as learned dead reckoning, i.e. to regress a local increment and integrate it, and to report the integrated path against insfilterMARG with ATE, RTE and mean quaternion error, rather than with a teacher-forced one-step MSE that hides drift. A secondary problem, which this document does not pretend to have solved, is that a recursive net which feeds back its own pose can look almost perfect while it is still being handed the ground truth, and can come apart the moment that crutch is removed.

## Hipótesis

A dual-head temporal convolutional network that maps a one-second window of MARG measurements and GPS velocity to a residual velocity and a 0.1 s axis-angle increment, with no ground-truth pose in the inputs, can be integrated into a TEST trajectory whose mean quaternion error is lower than that of insfilterMARG on the same stream, and whose ATE is at least comparable to open-loop integration of the GPS velocity itself.

## Objetivos

#### Objetivo General:

To design, train and evaluate a learned increment estimator that fuses MARG inertial measurements with GPS velocity, integrates the predicted increments into a pose, and compares that pose with a classical MARG/GNSS extended Kalman filter on the same simulated trajectory, without feeding the ground-truth previous state and without retrieving the filter’s Q and R by hand.

#### Objetivos Específicos:

The specific objectives are: 1) to generate a reproducible MARG-plus-GPS-velocity dataset from the LoggedQuadcopter trajectory and insfilterMARG, with the filter’s own pose logged as a baseline; 2) to train a dual-head temporal convolutional network on residual velocity and short-horizon orientation increments, using a chronological holdout; 3) to integrate those increments and report ATE, RTE and mean quaternion distance against the EKF and against GPS-velocity dead reckoning; 4) to document the recursive closed-loop alternative and the hybrid residual-on-EKF alternative, and to keep them out of the defended comparison table.

## Novedad científica, tecnológica o aportación

I do not claim a new class of neural filter. The increment idea is already in IONet, RIDI and RoNIN for IMU-only pedestrians [1]–[3]; what this work adds, and what I consider the actual contribution at master’s scope, is a MARG-plus-GPS-velocity increment model with a 3-D position and quaternion head, trained and scored against the same insfilterMARG instance that produced the sensor noise, with teacher-forced previous pose explicitly rejected. The recursive net that consumes its own last velocity and quaternion is the more vacant niche in the reading list, and it is reported as a closed-loop pilot, not as the defended method; the hybrid that adds a learned leftover to the EKF is documented because it is the pattern the papers already built [5]–[8], not because it is offered as novelty.

# ESTADO DEL ARTE o de la TÉCNICA

Resumen: This chapter walks from classical GNSS/INS fusion to learned inertial odometry and to the hybrids that keep a Kalman filter in the loop, and it closes on the three design options that were actually available for this thesis; the point of the survey is to show where the defended increment model sits, and why the most published hybrid pattern was not treated as the automatic “modern” choice.

## Classical GNSS/INS integrated navigation

The textbook solution for combining an inertial measurement unit with satellite observations is still a Kalman filter, in the linear case, or an extended Kalman filter once the attitude and the specific-force rotation make the dynamics nonlinear; Kalman stated the discrete predict/update recursion in 1960 [10], and Groves is the reference this work uses for frames, inertial error models, and the loosely versus tightly coupled GNSS/INS taxonomy [11]. In the loosely coupled picture the GNSS receiver produces a position and a velocity in a navigation frame, those observations update an INS that has been integrating accelerometer and gyroscope measurements, and the designer’s real labor is the pair of covariance matrices Q and R, i.e. how much the prediction is trusted versus how much the satellite fix is trusted.

A magnetic, angular-rate and gravity (MARG) unit adds a magnetometer to the usual six inertial axes, which gives the filter an absolute heading observation that a gyro-only integrator does not have; MathWorks’ insfilterMARG is an error-state EKF built for that sensor suite plus GPS, and it is the classical baseline of this thesis because it is also the generator of the simulated measurements. Over a 50 s excerpt of the official example the filter reaches position RMSE values on the order of half a meter per axis and a quaternion distance of about 1.45°; on the full LoggedQuadcopter log used here, roughly 142 s after warmup, the same filter, scored on the TEST slice with the same ATE/RTE protocol as the network, sits at 0.78 m ATE, 0.20 m RTE and 7.46° mean quaternion error. Those numbers are the opponent, not a footnote, and they already include GPS position (latitude, longitude, altitude) in the update, which the learned increment model is never given.

It is important to note that a well-tuned EKF is not a weak baseline, and that “learning replaces the filter” is only an interesting sentence if the comparison is run on the same stream, with the same holdout, and with metrics that punish drift rather than one-step pose MSE. The rest of this chapter is the set of published alternatives to writing Q and R by hand.

## Learning-based inertial odometry

The learning-based line starts from a simple observation, which Chen et al. put in the abstract of IONet: continuous integration is a curse, because a small bias becomes a walk, and the way out is to cut the inertial stream into independent windows and regress a local displacement [1]. RIDI had already treated the same instinct with shallow models, an SVM on phone placement and an SVR on gravity-aligned 2-D velocity, then a least-squares bias correction and double integration, and it left deep networks as explicit future work [2]. RoNIN turned that recipe into a large in-the-wild benchmark and three backbones, ResNet, LSTM and a temporal convolutional network, still IMU-only, still 2-D velocity, still without a filter at inference, and it is the paper from which this thesis takes ATE and RTE as the path metrics [3].

OriNet is the closest published cousin on the orientation side: a multi-LSTM plus MLP that regresses a quaternion from gyroscope samples, a time step, and the previous quaternion, and the authors themselves describe the net as acting like an EKF that updates from the last state [4]. The previous-quaternion input gave them a large gain over their own earlier model; it is also, if that previous quaternion is the ground truth, teacher forcing, i.e. the network is never asked to live with a heading it just got wrong. I consider that distinction more important than the architecture diagram, because a master’s table that reports a one-step quaternion MSE under teacher forcing is answering a different question from “can this thing fly.”

Almost all of the increment literature above is IMU-only, magnetometer disabled, no GPS, and trained on real pedestrian or MAV logs. The gap that is actually available, once teacher-forced previous pose is taken off the table, is narrower and cleaner: a full MARG suite plus GPS velocity, a 3-D increment and a quaternion, scored against insfilterMARG on the same simulation. That is not a new paradigm; it is a fair increment on [1]–[3] with a heading head that [4] already showed was learnable.

## Hybrid learning and filtering

Hybrid approaches combine hand-crafted algorithms and learning models [9]; in inertial navigation that sentence has a very concrete meaning, the Kalman filter stays, and the network is hired to do the part a human used to tune. TLIO regresses a 3-D displacement and an uncertainty from a 1-D ResNet and feeds both into a stochastic-cloning EKF [5]. AI-IMU keeps an invariant EKF and trains a small 1-D CNN only to adapt the pseudo-measurement noise, i.e. the network is the R schedule, not the state [6]. Al-Sharman trains a DNN on the Kalman filter’s own attitude so that residual noise can be subtracted from a filter that already ran [7]. Wang et al. push the same idea into GNSS outage: a Transformer–LSTM predicts pseudo-GNSS increments that a 15-state KF then consumes [8].

This is the developed 2020–2025 line, and it is also the easiest thesis to write if the only goal is to look contemporary. I thought, at the beginning of this work, that “put the LSTM on the EKF” was the sophisticated option; after reading [5]–[8] it became clear that it is the occupied option. A hybrid can still be a useful pilot, and one was implemented, but it cannot be sold as the novelty of this document.

## Deep learning for GNSS/INS outage aiding

Wang et al. are the closest GNSS/INS-plus-deep-learning analog in the reading list: the network sees a 12-D inertial and INS-derived window and predicts a 4-D GNSS pseudo-increment, which is a learned measurement, not a learned pose, and the filter remains the thing that closes the loop during a 50 s outage [8]. The headline reductions versus pure INS are large, as they should be when a satellite-like measurement is reintroduced; the methodological lesson for this thesis is different, namely that predicting a GNSS-like observation to feed a KF is already a published pattern, and repeating it with insfilterMARG would be an implementation of that pattern.

## Synthesis and the three design options

Chen et al.’s survey organizes localization into model-based, learning-based and hybrid families [9], and that trichotomy is exactly the fork this project had to pick from once the original teacher-forced previous-state network was discarded. Teacher forcing, in this document, means that during training the previous pose fed to the model is the ground-truth pose at t−1; the net never sees its own error, the loss is easy, and the number that comes out overstates in-flight drift. OriNet uses a previous quaternion in that spirit [4]. That setup is kept only as a diagnostic print, never as the claim.

Option A drops the previous pose entirely. The network sees MARG and GPS velocity, regresses a local increment, residual velocity plus a short-horizon axis-angle, and those increments are integrated; there is no Q and no R, and the comparison with insfilterMARG is a comparison of two estimators on the same measurements, except that the EKF is still allowed to use GPS position, which A is not. The idea is IONet/RoNIN-style learned dead reckoning [1], [3], extended to magnetometer, GPS velocity, and a quaternion.

Option B keeps a previous state, but it is the network’s own last prediction, and training is closed-loop, with scheduled sampling as the on-ramp from ground truth to self-generated pose [17]. The net is the filter; OriNet is the cousin, except OriNet teacher-forces a gyro-only quaternion and has no GPS [4]. Among the three options this is the least occupied niche in the paper set, and it is also the one whose wheels come off, which is why it is written as future work with a pilot, not as the main table.

Option C keeps insfilterMARG and trains a network on the leftover, a residual and a noise scale, or, in the papers, a covariance or a pseudo-GNSS increment [5]–[8]. Q and R come back unless the net replaces them. This is the safest implementation and the weakest novelty claim; the document records a C0 pilot so that the fork is empirical, not rhetorical, and it does not present C as the defended method.

The novelty ranking inside this reading list is therefore the inverse of the publication count: B is the most open and the hardest to train, A is a thinner but honest increment on IMU-only odometry, and C is the pattern that already has a venue trail. The choice, which Chapter 4 has to justify in engineering terms, is A for the main thesis, B for the recursive extension, and C as related work that was run once so the residual-on-EKF story would have numbers.

Figura 1. Ventana que se abre para insertar una imagen.

Tabla 1. Presentación de información en tablas.

| Distancia (km) | 2 | 3 | 5 | 7 | 8 | 10 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Precio ($) | 441,5 | 661,5 | 1.101,5 | 1.541,5 | 1.761,5 | 2.201,5 | 2.641,5 | 2.861,5 |

# MARCO TEÓRICO/CONCEPTUAL

Resumen: This chapter is a term-by-term account of the quantities, sensors, filters, networks and metrics that the rest of the document uses, so that Chapter 4 can name a residual velocity or a quaternion distance without stopping to define them; the tone is that of a glossary, not of a second state of the art, and each term is tied to the citation that this work actually leans on.

## Reference frames and attitude

Navigation is not done in a single set of axes, and mixing them is how a filter, or a network, writes a number that looks precise and points the wrong way; Groves is the reference used here for the geographic, navigation and body frames [11].

Geographic frame. A position on the Earth is written as latitude, longitude and altitude, often abbreviated LLA; GPS receivers report this frame, and insfilterMARG consumes it in the update. The defended network never sees LLA, only a velocity already expressed in the navigation frame, because a held latitude in the feature vector would turn absolute trajectory error into a contest the filter had already been given the answer to.

NED. The North–East–Down navigation frame is the working frame of this thesis: the first axis points north, the second east, and the third down, so that a positive increment in the third coordinate is a descent. Ground-truth position, GPS velocity and the EKF position in filterstate.csv are all stored in NED, following the MathWorks MARG example [18].

Body frame. The accelerometer, gyroscope and magnetometer measure in the vehicle body, i.e. in axes fixed to the IMU. A specific force has to be rotated by the current attitude before it can be integrated as a change of NED velocity; that rotation is the reason attitude and position cannot be learned as two unrelated regressions if one later wants a consistent path [11].

Euler angles. Roll, pitch and yaw are an intuitive three-number attitude, and they are also a poor regression target near pitch ±90°, where the representation is singular. This work does not train on Euler angles.

Quaternion. A unit quaternion q = (q_w, q_x, q_y, q_z) is a four-vector on the sphere that represents a rotation without that singularity; the product of two quaternions is another rotation, and the inverse of a unit quaternion is its conjugate [11], [4]. The ground-truth and EKF orientations in the dataset are stored in wxyz order. Composition of a previous attitude q with a small increment Δq is written

q_k = q_{k−1} ⊗ Δq_k    (1)

and the increment itself is obtained, in this work, from a three-dimensional axis-angle vector v = θ u, with θ the rotation angle and u a unit axis, by the map

Δq = (cos(θ/2), u sin(θ/2))    (2)

Quaternion distance. Because q and −q are the same rotation, a useful scalar error is the geodesic angle used by OriNet and kept in Chapter 5 [4],

θ_D = 2 arccos(|q̂ · q|)    (3)

reported in degrees as a mean over the TEST sequence. A 180° value that never moves, as in the first increment experiment of this project, is a sign that the network has collapsed to an antipodal quaternion, not that the vehicle flipped.

## Inertial and satellite sensors

IMU. An inertial measurement unit, in the six-axis sense, is an accelerometer plus a gyroscope; the accelerometer reports specific force, i.e. acceleration minus gravity in the body frame, and the gyroscope reports angular rate. Integrating those signals is dead reckoning, and it walks off in seconds on a consumer IMU because of bias, noise density and scale-factor error [11].

MARG. A magnetic, angular-rate and gravity unit adds a magnetometer to the IMU, which gives an absolute heading observation that a gyro-only integrator does not have. The simulated sensor in this work is MATLAB’s imuSensor in the accel-gyro-mag configuration, with the noise densities and constant biases of the official example [18]; the twelve-channel learning window is accelerometer (3), gyroscope (3), magnetometer (3) and GPS velocity (3).

Sensor noise, bias and density. The example IMU is not a white-noise toy: the accelerometer and gyroscope carry a constant bias and a noise density, the magnetometer carries a large constant bias, and those parameters are also used to seed the EKF state. I consider that this is why a Q/R-free network is interesting on this log, and also why it is not a claim about an unseen physical IMU: the noise the net sees is the noise the example already wrote down.

GNSS and GPS velocity. A global navigation satellite system receiver, here a simulated gpsSensor standing in for a consumer GPS, produces a geodetic position and a velocity at 1 Hz, with a horizontal and vertical position accuracy of 1.6 m and a velocity accuracy of 0.1 m/s in the generator script. Velocity is zero-order held into the 160 Hz table so that the network always sees a 12-D sample; position LLA is withheld from that table and reserved for the EKF update. Open-loop integration of the held GPS velocity is the dumb baseline of Chapter 5, i.e. what the residual head must beat or tie before anyone talks about beating the filter.

Zero-order hold (ZOH). A 1 Hz measurement that is repeated until the next fix, so that a 160 Hz learner has a value on every row. ZOH of magnetometer and GPS velocity is used; ZOH of LLA is not, for the reason already stated.

## Classical sensor fusion

The classical estimator in this document is an extended Kalman filter. Kalman stated the linear recursion in 1960 [10]; Groves is the navigation reading [11]; insfilterMARG is the concrete MARG-plus-GPS implementation that both generates the comparison pose and, in the hybrid pilot, supplies the leftover that a network may try to subtract [18].

State-space model. A discrete process and a measurement are written

x_k = f(x_{k−1}, u_k) + w_k,    z_k = h(x_k) + v_k    (4)

where w_k is process noise with covariance Q and v_k is measurement noise with covariance R. The designer’s labor, when there is no network, is those two matrices.

Kalman filter. In the linear case the predict step propagates the mean and the covariance, and the update step computes a gain K that blends the prediction with the measurement [10],

x̂_{k|k−1} = F x̂_{k−1|k−1},    P_{k|k−1} = F P_{k−1|k−1} Fᵀ + Q    (5)

K_k = P_{k|k−1} Hᵀ (H P_{k|k−1} Hᵀ + R)⁻¹    (6)

x̂_{k|k} = x̂_{k|k−1} + K_k (z_k − H x̂_{k|k−1})    (7)

Q and R. Q says how much the prediction is trusted; R says how much the sensor is trusted. Swapping their names in a write-up, or putting a magnetometer variance into R as if it were already an angle, is a known way to get a filter that runs and a text that does not match the code. This thesis does not retune Q or R; the example values travel with the simulated IMU.

EKF and insfilterMARG. An extended Kalman filter linearizes f and h about the current estimate. insfilterMARG is an error-state EKF for a 22-dimensional MARG-plus-GPS state, including attitude, NED position and velocity, inertial biases and the geomagnetic vector [18]. On the TEST slice used in Chapter 5 it reaches 0.78 m ATE, 0.20 m RTE and 7.46° mean quaternion error. Those numbers already include GPS LLA in the update.

Loosely versus tightly coupled fusion. A loosely coupled GNSS/INS filter consumes the receiver’s position and velocity solutions; a tightly coupled filter consumes raw or processed satellite observables [11]. The baseline of this work is loosely coupled in that sense: fusegps is called with LLA and velocity. The learned estimator is looser still: it sees only the velocity channel.

## Deep learning models used in this work

The defended estimator is a temporal convolutional network with two linear heads. The terms below are only those that appear in that pipeline or in the alternatives of Chapter 2; Goodfellow et al. are the textbook backdrop [12].

MLP. A multilayer perceptron is a stack of affine maps and pointwise nonlinearities; here each head is a two-layer MLP, 128–64–3, on top of a pooled TCN body [12].

ReLU. The rectified linear unit, max(0, x), is the nonlinearity inside the TCN blocks and the heads.

Conv1d. A one-dimensional convolution slides a kernel along time, across channels; it is the natural layer for a window of IMU samples, and it is the stem of both the increment model and the older Conv1d–LSTM course network [15], [12]. A kernel of length 5 and 64 channels is the stem used in A8q.

TCN. A temporal convolutional network is a stack of dilated causal, or here residual, 1-D convolutions whose receptive field grows exponentially with depth. RoNIN already used a TCN as one of three inertial backbones [3]. The A8q body has four residual blocks with dilations 1, 2, 4 and 8; the representation fed to the heads is the concatenation of a global average over time and the last time step.

Residual velocity head. The position-related output is not a NED position and not a raw velocity, it is a correction δv such that

v̂ = v_GPS + δv    (8)

so that a network that outputs zero still reproduces GPS-velocity dead reckoning. The 0.1 s orientation head outputs the axis-angle of (2), not a unit quaternion, because a 160 Hz Δq is almost the identity and was observed to collapse to 180° in the first increment run.

LSTM. A long short-term memory cell is a recurrent unit with input, forget and output gates [13]. It is the backbone of IONet and OriNet [1], [4], and of the course-project network that this thesis retired; it is not the frozen A8q body. It is defined here because Chapter 2 cannot be read without it.

Dual head. Two small MLPs on a shared trunk, one for δv and one for the axis-angle. A single six-dimensional head was tried; ATE pressure spent the orientation. I consider that split the one architectural decision that actually changed the table, more than the width of the TCN.

## Training, losses and optimization

Train / validation / test split. Windows are extracted every 0.1 s from a one-second context. The log is cut chronologically, about 70 / 15 / 15 with a one-second gap, so that TRAIN, VAL and TEST do not share a hop; TEST is 121–141 s. This is a same-path holdout, not a claim that the net generalizes to another flight [12].

Normalization. Feature-wise and target-wise means and standard deviations are computed on TRAIN only and applied to VAL and TEST; without that scaling the position term dominates a joint loss and the orientation is under-learned.

MSE. Mean squared error on the standardized residual velocity and on the axis-angle is the training loss of A8q [12]. A geodesic loss on (3) was left as a possible ablation and was not used for the frozen row.

Adam. The optimizer is Adam at 10⁻³ [14], with minibatches of 256 and eighty epochs. Early stopping, in practice, is a validation checkpoint, and the checkpoint metric decides which compass one keeps.

Dropout. Randomly dropping units during training as a regularizer [16]. It is cited because Al-Sharman uses it and because the course network sketched it; A8q does not rely on it as the main control of overfitting, the chronological split and the dual head do more of that work.

## Dead reckoning, teacher forcing and scheduled sampling

Dead reckoning. Estimating the present pose from the last pose and a measured or predicted increment, with no external fix. Inertial integration is dead reckoning; IONet’s sentence about breaking continuous integration is the learning version of the same idea [1]. The defended estimator is learned dead reckoning plus a GPS-velocity measurement, not a pose prior.

Teacher forcing. During training, the previous state fed to a sequential model is the ground truth, not the model’s own last output. The loss becomes easy and the number overstates closed-loop drift; OriNet’s previous quaternion is in that family if that quaternion is the true one [4]. Teacher forcing is rejected as the claim of this thesis and kept only as a diagnostic column in the recursive pilot.

Exposure bias. The mismatch between a teacher-forced train-time input and a self-generated test-time input. B0 made that mismatch theatrical: a six-centimeter teacher-forced ATE and a 2.57 m closed-loop ATE, because the net had been looking up a NED position.

Scheduled sampling. Bengio et al.’s on-ramp: the probability of feeding ground truth is annealed toward zero so that the net is gradually asked to live with its own pose [17]. B1 uses a linear anneal over the first 40% of epochs and then stays at zero.

Closed-loop evaluation. The first TEST pose is taken from ground truth; after that the estimator consumes only sensors and its own last state. A is closed-loop in the integrator, not in the network, because A has no previous-state input. B is closed-loop in both.

## Evaluation metrics

ATE. Absolute trajectory error, taken from RoNIN’s evaluation culture [3]: the RMSE of the integrated NED position against ground truth on the 1 s TEST hops,

ATE = √( (1/N) Σ_i ||p̂_i − p_i||² )    (9)

A constant 0.09 m/s velocity bias becomes about a meter over a 20 s holdout, so ATE punishes drift. The EKF’s ATE is not a fair fight with A, because the filter sees LLA.

RTE. Relative trajectory error over a short window, here one second: the mean magnitude of the difference between estimated and true displacements on that hop [3]. A path that is translated but the right shape can have a bad ATE and a decent RTE. A8q’s claim against GPS-velocity integration is an RTE claim, 0.31 m versus 0.45 m.

RMSE. Root-mean-square error, the scalar cousin of ATE when the quantity is not a trajectory, e.g. a per-axis velocity or a one-step increment.

Those twelve numbered relations, and the terms around them, are the vocabulary of Chapters 4 and 5; what is not defined here, a Transformer, an invariant EKF, a stochastic-cloning filter, is named in Chapter 2 and left there.

# DESARROLLO METODOLÓGICO

Resumen: This chapter records the design fork, the simulated dataset, and the defended increment pipeline; the recursive and hybrid estimators are specified only as far as needed to make the later numbers readable, they are not a second and third method competing for the same claim.

## Design alternatives for the learned estimator

Option A is the defended estimator. A one-second window of accelerometer, gyroscope, magnetometer and GPS velocity, twelve channels, no pose, is mapped to a residual 3-D velocity and a 0.1 s axis-angle; position is integrated every second as pos ← pos + v̂·1.0, and the quaternion is integrated every 0.1 s as q ← q ⊗ quat(Δθ). There is no process-noise table and no measurement-noise table. I consider this the only option that keeps the original taste of the project, i.e. that everything the estimator needs is learned, and that still has a clean published ancestor in windowed increment odometry [1]–[3].

Option B is a learned recursive filter. The same window is concatenated with the last velocity and the last quaternion produced by the net, scheduled sampling anneals the probability of feeding ground truth from one to zero, and TEST is seeded once and then never sees ground truth again [17]. Feeding the absolute NED position was tried and discarded: with the true position in its hand the teacher-forced score collapsed to centimeters, which is a lookup of the training trajectory, not a filter. B is the option that still honors “no Q/R” and is not already the plot of [5]–[8]; it is also the option that can look healthy on a one-second unroll and sick on a twenty-second flight, which is why it is not the main-table method.

Option C is the hybrid. insfilterMARG is left frozen, including the example’s AccelerometerNoise, Rpos and Rvel, and a network predicts the leftover error μ and a log-standard-deviation, trained with a Gaussian negative log-likelihood; the corrected pose is the EKF pose plus μ. That is Al-Sharman on this filter, and it is a stand-in for “the model is the noise” without rewriting Q in MATLAB and rerunning the example [6], [7]. A full AI-IMU loop is not claimed.

The decision was A. The argument is not that A is the most novel, B is, and it is not that A is the easiest to make look good on ATE, C is, because C inherits GPS latitude, longitude and altitude through the filter. A is the option that can be compared with the EKF without pretending to be the EKF, that does not require a previous true pose at runtime, and that still lets B and C be written down as the recursive extension and the already-published hybrid. Teacher forcing remains a diagnostic column in B, so the exposure-bias gap is visible, and it is not sold as a result.

## Simulated dataset and the classical baseline

The dataset is generated in MATLAB from LoggedQuadcopter.mat, using imuSensor and gpsSensor at 160 Hz and 1 Hz, and insfilterMARG as both the noise-consistent classical estimator and the source of the filter-state log. Magnetometer and GPS velocity are zero-order held at 1 Hz in the learning table; GPS latitude, longitude and altitude are consumed by the EKF and are not written into the twelve-channel stream, because putting a held LLA into the network would turn ATE into a competition the EKF had already been given the answer to. The first second of warmup is dropped. Ground-truth position is in NED; ground-truth orientation is a unit quaternion in wxyz order.

Windows are one second long, extracted every 0.1 s, which yields on the order of 1400 windows on the 142 s log. The split is chronological, 70 / 15 / 15 with a one-second gap, so that TRAIN occupies roughly 1–97 s, VAL 99–120 s and TEST 121–141 s; calling this “generalization” would be dishonest, it is a same-path holdout. Feature and target statistics are computed on TRAIN only. The EKF pose used by C and by the baseline columns is read from filterstate.csv, aligned to the same window-end samples.

## Defended estimator: increment TCN

The backbone is a temporal convolutional network rather than the course-project Conv1d–LSTM: a 12-to-64 stem, four residual blocks with kernel 5 and dilations 1, 2, 4 and 8, then a concatenation of global average pooling and the last time step, 128 units, and two heads, each 128–64–3, one for residual velocity and one for the 0.1 s axis-angle. A shared six-dimensional head was tried; position pressure spent the orientation, which is why the heads were split. The residual form is v̂ = v_gps + δv, so that a network that outputs zero still reproduces GPS-velocity dead reckoning, and any ATE gain has to be earned against that baseline, not against a cold start.

Training uses Adam at 10⁻³, minibatches of 256, eighty epochs, and a validation checkpoint. Early runs that checkpointed on ATE alone, or on a sum ATE + quat/30, kept the path and discarded the compass; the frozen weights, A8q, are the quaternion checkpoint of the same dual-head run, not a second architecture. Targets are standardized. The loss is mean squared error on the normalized residual velocity and on the axis-angle; a geodesic quaternion loss was left as a possible A-side ablation and was not used for the frozen row.

Integration at evaluation is the same for A and for the GPS-velocity baseline: the first TEST pose is taken from ground truth, predicted 1 s mean velocities are applied on the ten-sample hop, and 0.1 s rotations are applied on every extract. ATE is the RMSE of the integrated NED position against ground truth on those 1 s samples; RTE is the mean magnitude of the 1 s relative displacement error; quaternion error is the mean of 2 arccos(|q̂ · q|), in degrees, on the 0.1 s sequence [3], [4]. The EKF is scored on the same timestamps. GPS-velocity ATE on this TEST slice is 1.10 m, RTE 0.45 m; those two numbers are what “the network did something” has to beat, or at least tie, before anyone talks about beating the EKF.

## Recursive and hybrid pilots

The B1 pilot reuses the A dataset and the same heads. Previous state is last velocity plus last quaternion, not NED position; scheduled sampling falls from 1 to 0 over the first 40% of epochs and stays at 0; training unrolls 40 hops, 4 s, and TEST is closed-loop from the first ground-truth pose. The C0 pilot concatenates the frozen EKF pose to the same twelve-channel window, predicts μ and log σ, and applies the mean residual independently at each window. Neither pilot retunes MATLAB Q or R. Their numbers belong in Chapter 6 and in the related-work discussion, not as replacements for A8q.

Three estimators can be built from the same MARG-plus-GPS-velocity window and the same LoggedQuadcopter log, and they are not three inventions, they are three answers to a single question: what is allowed to sit in the previous-state slot. The rejected answer is ground-truth pose at t−1. The remaining answers are: nothing (A), the network’s own last prediction (B), and the EKF’s pose (C).

# RESULTADOS Y DISCUSIÓN

Resumen: This chapter reports the increment-model ablations that produced the frozen A8q row, compares that row with insfilterMARG and with GPS-velocity integration on the same TEST slice, and then, as discussion rather than as a second defended method, records the closed-loop recursive pilot and the hybrid leftover-error pilot.

## Resultados

The TEST slice is the last twenty-one seconds of the log, 121–141 s, twenty-one one-second hops after a one-second gap from validation. All rows below use that slice, the same ground truth, and the same ATE / RTE / mean quaternion protocol. A8q is the frozen dual-head TCN, quaternion checkpoint; the A8 ATE checkpoint is shown only as the trade-off that was rejected, a better path with a dead compass. GPS-vel is not a second network, it is pos ← pos + v_gps·1.0 from the first TEST pose.

Tabla 2. TEST metrics, 121–141 s, versus ground truth. A8q is the defended row. B1 and C0 are pilots.

| Estimator | ATE (m) | RTE (m) | Quat (deg) |
| --- | --- | --- | --- |
| A8q (defended) | 1.12 | 0.31 | 4.95 |
| A8 ATE-ckpt (rejected) | 0.93 | 0.32 | 14.3 |
| GPS-vel integrate | 1.10 | 0.45 | — |
| insfilterMARG | 0.78 | 0.20 | 7.46 |
| B1 closed-loop (pilot) | 1.42 | 0.30 | 2.09 |
| C0 EKF+μ (pilot) | 0.89 | 0.29 | 5.44 |

A0–A2 established that a 160 Hz unit quaternion increment is almost the identity and that a one-second axis-angle is too coarse; the usable orientation target is a 0.1 s rotation, and the usable position target is a one-second mean velocity, later a residual on GPS velocity. A3–A7 showed the other failure mode: when velocity and rotation share one head, any ATE pressure spends the quaternion, and a one-second hop on a 142 s log starves the train set down to 97 windows. A8 split the heads and, for the first time, put ATE below GPS-vel on the ATE checkpoint, 0.93 m, at the cost of 14.3°; the quaternion checkpoint of that same run is A8q, 1.12 m ATE, 0.31 m RTE, 4.95°. A9 was a retrain of the same recipe and did not beat it, which is why A8q is frozen rather than re-rolled.

## Discusión

B1 is the first recursive run that is not a pose lookup. Closed-loop TEST is 1.42 m ATE, 0.30 m RTE and 2.09°, which is the best attitude in the project and a worse path than A8q; teacher-forced B1 is 1.26 m and 0.12°, an honest gap, unlike B0’s 6 cm teacher score that had been reading the NED position. That is the Chapter 6 sentence: a net that sees its last quaternion can beat A on heading and still lose the field.

C0 added a learned leftover to the frozen EKF and produced 0.89 m ATE, 0.29 m RTE and 5.44°. The attitude is better than the raw filter and worse than A8q; the path is worse than the filter it was sitting on, because the residual moved the EKF by about 0.7 m per sample and did so independently at each window. It can also look, carelessly, like a win over A8q on ATE; that comparison is the LLA leak again. The hand-tuned Q and R were already good on this log. This is why C stays a literature pilot.

The headline I am willing to defend is therefore narrow: on this simulated quadcopter, with this split, a dual-head increment TCN that never sees pose can beat insfilterMARG on orientation, tie GPS-velocity integration on ATE, and beat it on RTE, and the tempting alternatives, teacher-forced pose, recursive pose, and EKF residual, either lie, help heading at the expense of the path, or smear a filter that was already the position winner.

The difference between A8q and the EKF is not a mystery once the observations are written down. The filter sees GPS position; A8q does not; 0.78 m versus 1.12 m ATE is therefore not a fair fight, and I do not read it as the network losing a contest it was invited to. RTE is the fairer path metric, 0.31 m versus 0.20 m, close and still behind. Quaternion is the number A8q actually wins, 4.95° versus 7.46°, and it is also the number the ATE checkpoint threw away. Against GPS-vel the story is the opposite: ATE is a tie, 1.12 m versus 1.10 m, and RTE is a win, 0.31 m versus 0.45 m, i.e. the residual head did not invent a better mean speed than the satellite, it made the hops the right shape. A leftover velocity bias of about 0.09 m/s is still enough to walk a meter over the twenty-second holdout, which is why the last ten seconds are worse than the first, 1.41 m versus 0.74 m, and why I do not claim that ATE is solved.

# CONCLUSIONES

Resumen: This chapter restates what was asked of the increment estimator, what was actually shown on the simulated holdout, and which of the two unused forks, the recursive net and the hybrid leftover, are worth another student’s time.

## Conclusiones

The general objective was met in the limited sense that a learned increment estimator exists, it does not consume ground-truth pose, it does not retrieve Q and R, and it has a TEST row against insfilterMARG. The quaternion half of the hypothesis holds on this log, 4.95° versus 7.46°; the ATE half holds only as a tie with GPS-velocity integration, not as a victory over the EKF, and I think that is the honest way to write it. The specific objectives on the dataset, the dual-head TCN, and the ATE/RTE protocol were carried through; the fourth objective, to document B and C without promoting them, is the reason those rows are not in the main claim. A trade-off remains, the more the checkpoint hunts ATE the faster the compass dies, or, conversely, the snapshot that keeps heading pays about twenty centimeters of path versus the ATE specialist, and versus the EKF it pays more than that because the filter still owns GPS position. The work is simulated, single-trajectory, same-path chronological holdout; those are limitations, not footnotes.

## Trabajo Futuro

The recursive estimator is the line I would actually continue. B1 already shows that feeding last velocity and last quaternion, with scheduled sampling and a four-second unroll, can put closed-loop heading at 2.09° while giving back three tenths of a meter of ATE; the next useful B is a longer TEST, more than one flight, and, if anyone insists, a geodesic orientation loss, not another NED-position input. The hybrid leftover is not a priority, C0 over-corrected a filter whose Q and R were already adequate; a true AI-IMU rerun, new noise parameters written back into insfilterMARG, would only be interesting on a sensor that is not the example IMU. The hardware log from the soccer field is the other obvious extension, and it would force a ground-truth discussion that the geometric field model of the earlier project never settled. Several random seeds on A8q would also be more valuable than another architecture, A9 already showed that one retrain can miss the frozen row.

BIBLIOGRAFÍA

[1] C. Chen, X. Lu, A. Markham, and N. Trigoni, "IONet: Learning to Cure the Curse of Drift in Inertial Odometry," in Proc. AAAI Conf. Artif. Intell., 2018.

[2] H. Yan, Q. Shan, and Y. Furukawa, "RIDI: Robust IMU Double Integration," in Proc. Eur. Conf. Comput. Vis. (ECCV), 2018, pp. 621–636.

[3] S. Herath, H. Yan, and Y. Furukawa, "RoNIN: Robust Neural Inertial Navigation in the Wild: Benchmark, Evaluations, and New Methods," in Proc. IEEE Int. Conf. Robot. Autom. (ICRA), 2020, pp. 3146–3152.

[4] M. A. Esfahani, H. Wang, K. Wu, and S. Yuan, "OriNet: Robust 3-D Orientation Estimation With a Single Particular IMU," IEEE Robot. Autom. Lett., vol. 5, no. 2, pp. 399–406, Apr. 2020.

[5] W. Liu et al., "TLIO: Tight Learned Inertial Odometry," IEEE Robot. Autom. Lett., vol. 5, no. 4, pp. 5653–5660, Oct. 2020.

[6] M. Brossard, A. Barrau, and S. Bonnabel, "AI-IMU Dead-Reckoning," IEEE Trans. Intell. Veh., vol. 5, no. 4, pp. 585–595, Dec. 2020.

[7] M. K. Al-Sharman, Y. Zweiri, M. A. K. Jaradat, R. Al-Husari, D. Gan, and L. D. Seneviratne, "Deep-Learning-Based Neural Network Training for State Estimation Enhancement: Application to Attitude Estimation," IEEE Trans. Instrum. Meas., vol. 69, no. 1, pp. 24–34, Jan. 2020.

[8] J. Wang, S. Xu, Y. Xing, J. Lu, and H. Hu, "A Pseudo-Increment Prediction Method for GNSS/INS Integrated Navigation Based on Transformer–LSTM During GNSS Outages," in Proc. IEEE VTC2025-Fall, 2025.

[9] C. Chen, B. Wang, C. X. Lu, N. Trigoni, and A. Markham, "A Survey on Deep Learning for Localization and Mapping: Towards the Age of Spatial Machine Intelligence," arXiv:2006.12567, 2020.

[10] R. E. Kalman, "A New Approach to Linear Filtering and Prediction Problems," J. Basic Eng., vol. 82, no. 1, pp. 35–45, 1960.

[11] P. D. Groves, Principles of GNSS, Inertial, and Multisensor Integrated Navigation Systems, 2nd ed. Norwood, MA, USA: Artech House, 2013.

[12] I. Goodfellow, Y. Bengio, and A. Courville, Deep Learning. Cambridge, MA, USA: MIT Press, 2016.

[13] S. Hochreiter and J. Schmidhuber, "Long Short-Term Memory," Neural Comput., vol. 9, no. 8, pp. 1735–1780, 1997.

[14] D. P. Kingma and J. Ba, "Adam: A Method for Stochastic Optimization," in Proc. ICLR, 2015.

[15] Y. LeCun, L. Bottou, Y. Bengio, and P. Haffner, "Gradient-Based Learning Applied to Document Recognition," Proc. IEEE, vol. 86, no. 11, pp. 2278–2322, 1998.

[16] N. Srivastava, G. Hinton, A. Krizhevsky, I. Sutskever, and R. Salakhutdinov, "Dropout: A Simple Way to Prevent Neural Networks from Overfitting," J. Mach. Learn. Res., vol. 15, pp. 1929–1958, 2014.

[17] S. Bengio, O. Vinyals, N. Jaitly, and N. Shazeer, "Scheduled Sampling for Sequence Prediction with Recurrent Neural Networks," in Proc. Adv. Neural Inf. Process. Syst. (NeurIPS), 2015.

[18] The MathWorks, Inc., "insfilterMARG: Estimate pose using MARG and GPS data," Sensor Fusion and Tracking Toolbox documentation, Natick, MA, USA.

[Bibliografía Estilo IEEE: http://www.ieee.org/documents/ieeecitationref.pdf

El orden de la bibliografía debe ser el orden en el que aparece en el documento. Toda bibliografía puesta en esta sección deberá estar citada dentro del texto. La forma de hacer la cita cuando es una sola es [1], si son varias y salteadas es [3], [5], [8]. Si son varias consecutivas es [5]-[8]. Combinadas es [2]-[6], [9].

El formato de la bibliografía es como lista numerada simple, sin tabla.

El formato de cada una es como se describe a continuación y dependiendo del tipo:

[Book Article

- For an article in an edited book, use practice similar to that for author w/ editor or translator above, inserting article title between author[s] names and book title.

- [Citation Number] Author name[s], "article title," in  book title, editor names, publication location: publisher, year, pages.

- Examples:]

E.D. Lipson and B.D. Horowitz, "Photosensory reception and transduction," inSensory Receptors and Signal Transduction, J.L. Spudich and B.H. Satir, Eds.  New York: Willey-Liss, 1991.  pp. 1-64.

- J. Lacan.  "The insistence of the letter in the unconscious,"  in Psychoanalysis and Language, David Lodge, Ed., J. Rose, Trans.,  Ithaca, NY: Cornell University Press, 1992, pp. 123-34.

[Journal

- [Citation Number] Author name[s], "article title,"journal title , volume number, issue number, month (abbrv.), pages, publication year.

- Only include information which is pertinent to your source.  For example, many professional and academic journals do not have an issue month.  In that case, or when it seems unnecessary, do not include it in your citation.

- Examples:]

K.A. Nelson, R.J. Dwayne Miller, D.R. Lutz, and M.D. Fayer,  "Optical generation of turntable ultrasonic waves," Journal of Applied Physics, vol. 53, no. 2, Feb., pp. 1144-1149.

J. Allemang, "Social studies in gibberish," Quarterly Reviews of Doublespeak, vol. 20, no. 1, pp. 9-10.

[Popular Periodical Article (monthly or bimonthly)

- In the case of popular monthly or bimonthly periodicals, omit volume number and issue, identifying instead by month and year of publication.

- [Citation Number] Author name[s], "article title,"periodical  title , month (abbrv.), pages, publication year.

- Examples:]

J. Fallows, "Network technology," Atlantic Monthly, Jul.,  pp. 34-36, 1994.

[Popular Periodical Article (Biweekly, weekly, or daily)

- In the case of more frequently published periodicals, use day, month, and year to identify.

- [Citation Number] Author name[s], "article title,"periodical  title, day number month (abbrv.), pages, publication year.

- Examples:]

B. Metcalfe, "The numbers show how slowly the Internet runs today," Infoworld, 30 Sep., p. 34, 1996.

J. Turner, "Disorder 'kills without warning,'"  The Toronto Star, 26 Jun., pp. F1-F2, 1998.

[Paper Published in Conference Proceedings or Presented at Conference

- Treat a presentation in conference proceedings like an article in an edited book, including all available publication information.  Conference proceedings are often published by the organization holding the conference; in that case, do not cite the publisher.

- Example:]

Paez-Borrallo, I.A. Perez-Alavarezz, and S.Z. Bello,  "Adaptive foltering in data communications with self improved error reference," in Proc. IEEE ICASSP '94, 1994, pp. 65-68.

[Treat an unpublished paper presented as a conference in the following manner: [Citation Number] Author name[s], "article title," presented at conference title. conference location, year.

- Example:]

M. Lai, B. Chen, and S. Yuan,  "Toward a new educational environment," presented at 4th Int. World Wide Web Conf. Boston, MA, 1995.

[Web Page

- Give the author, title, type of medium (enclosed in brackets), volume and issue number (if on-line journal), page number (if relevant or given), and the year and the month of publication (in parentheses).  Then give the full internet address or the name of the online service provider prefaced by "Available at ".  If not an on-line journal, also put [cited year month day] before "Available at".

- Examples:]

A. Harnack and G. Kleppinger, "Beyond the MLA Handbook: Documenting Electronic Sources on the Internet." Kairos, [Online serial] 1 (2), (1996 Sum), Available at HTTP: http://english.ttu.edu.kairos/1.2/

P. Curtis, "Mudding: Social Phenomena in text-based virtual realities," [Online document] Aug. 1992, [1996 Aug 30], Available at FTP: parcftp.xerox.com/pub/MOO/papers/DIAC921992.

********El estilo es Referencia]

APÉNDICE A. Título

APÉNDICE B. Título